Emergency call channel: Tap to select for emergency calls.

Transmit Key: Hold down for 1 sec before speaking

Lock Button and Icon:

Tap on/off to change channel

Tuning dial: Rotate when selecting new channel

Power & Volume dial:

Rotate to turn on/off & adjust volume

Battery indicator: If low, place on charger during practice (NOT overnight)

BASIC CARE AND USE:

- Do not carry the radio by the antenna.

- Turn on radio and confirm the channel and battery level before launching.

- BBOP will use either channel 68 or channel 71, depending on radio traffic

- To change the channel:

  - Unlock the radio by tapping the lock button

  - Hold the blue 16/C button for 1 sec until channel indicator blinks

  - Hold the blue 16/C button for 3 sec until two short beeps are heard

  - Rotate the tuning dial to the desired channel

  - Finish by pushing the 16/C button to select the call channel

  - Re-lock the radio by tapping the lock button, confirmed by the appearance of the lock icon.

- If the radio requires charging, place on the charger for the duration of practice only.

- After use, rinse the radio thoroughly with freshwater and dry.

IN CASE OF EMERGENCY:

Contact other vessels and the Coast Guard by sending a Distress call on Channel 16. Use this procedure for your own crew as well as for other vessels.

USING CHANNEL 16

Push the blue 16/C button to select the emergency channel and transmit the following:

- Press and hold the transmit button.

“MAYDAY MAYDAY MAYDAY”

- Say your call sign and a brief description of the vessel.

“THIS IS 6-PERSON CANOE, CALL SIGN OC6. MAYDAY, OC6”

- State your location simply and clearly.

“LOCATED AT …” E.G. NEAR SHORE, 1 MILE SOUTH OF POST POINT

- State the nature of your emergency and assistance required.

”MEDICAL EMERGENCY FOR [#] PERSON/S WITH …”

EXAMPLES:

“MEDICAL EMERGENCY FOR 1 PERSON WITH SEVERE BLEEDING”

“MEDICAL EMERGENCY FOR 2 PERSONS IN THE WATER”

“VESSEL OVERTURNED AND SINKING. 6 SOULS IN THE WATER”

- When a response is received, give any other information that might facilitate the rescue. Answer each question directly. Be clear and concise. Continue to use call signs.

EXAMPLES:

- “OC6 TO Terrapin: WE ARE DRIFTING 25 YARDS OFF THE TRAIN TUNNEL, OVER”

- “OC6 TO COAST GUARD: CONFIRMING RENDEVOUZ AT RED CHANNEL MARKER, OVER”